# Off-Grid Refrigeration System Sizing Worksheet

Calculate Your Solar, Battery, and Component Requirements

Instructions: Fill in the white cells with your specific information. The worksheet will guide you through calculating your complete off-grid refrigeration system requirements. Use a pencil or make copies of this worksheet to try different scenarios.

## Step 1: Determine Your Refrigeration Power Needs

List all refrigeration appliances you plan to power:

| Appliance | Watts | Hours/Day | Wh/Day | Notes |
| --- | --- | --- | --- | --- |
| Refrigerator/Freezer #1 |  |  | = W × H |  |
| Refrigerator/Freezer #2 |  |  | = W × H |  |
| Additional Appliance |  |  | = W × H |  |
| TOTAL DAILY CONSUMPTION (Summer) |  |  | _____ Wh |  |

Important: The 'Hours/Day' for refrigerators should be the actual runtime (duty cycle × 24 hours), not 24 hours. Measure with a Kill-A-Watt meter or use manufacturer specifications.

### Environmental Adjustment

| Summer consumption (from above): | _____ Wh/day |
| --- | --- |
| Multiply by 1.25 for hot summer conditions: | _____ Wh/day |

## Step 2: Choose Your System Voltage

Check one (recommendation: 12V for <500Wh/day, 24V for 500-2000Wh/day, 48V for >2000Wh/day):

| ☐ 12V | Best for small systems, high amp draw, thick wires needed |
| --- | --- |
| ☐ 24V | Recommended for most homestead systems, balanced efficiency |
| ☐ 48V | Best for large systems, lowest amp draw, most efficient |

Calculate daily amp-hours (Ah):

| Daily consumption (Wh/day) ÷ System voltage (V): | _____ Ah/day |
| --- | --- |

## Step 3: Size Your Battery Bank

Determine days of autonomy (recommended: 2-3 days minimum for refrigeration):

| Desired days of autonomy: | _____ days |
| --- | --- |

Select battery type:

| ☐ Lead-Acid | 50% usable depth of discharge (cheaper, requires maintenance) |
| --- | --- |
| ☐ Lithium | 80% usable depth of discharge (more expensive, longer life, no maintenance) |

Calculate required battery capacity:

| Daily Ah × Days of autonomy: | _____ Ah |
| --- | --- |
| Depth of discharge (0.50 for lead-acid, 0.80 for lithium): | _____ |
| REQUIRED BATTERY CAPACITY (Ah ÷ DoD): | _____ Ah |

## Step 4: Size Your Solar Array

Determine your location's peak sun hours (use worst-case winter months):

| Peak sun hours (winter): | _____ hours |
| --- | --- |

Tip: Search online for 'peak sun hours [your city]' or use PVWatts calculator at pvwatts.nrel.gov

Calculate required solar capacity:

| Daily consumption (Wh/day): | _____ Wh |
| --- | --- |
| Peak sun hours: | _____ hours |
| Wh ÷ hours = minimum solar watts: | _____ W |
| System efficiency factor (use 0.75): | 0.75 |
| REQUIRED SOLAR WATTS (W ÷ 0.75): | _____ W |

Recommended: Add 20-50% extra capacity for weather variability and system degradation over time.

## Step 5: Select Charge Controller

Calculate required charge controller capacity:

| Total solar watts: | _____ W |
| --- | --- |
| System voltage: | _____ V |
| Watts ÷ Volts = required amps: | _____ A |
| CHARGE CONTROLLER (A × 1.25 safety margin): | _____ A |

Recommended: Choose MPPT controller over PWM for 20-30% better energy harvest, especially in cold weather.

## Step 6: Inverter Sizing (If Using AC Appliances)

Note: DC refrigerators are more efficient and don't require an inverter. If using AC appliances, size inverter for surge current.

| Refrigerator running watts: | _____ W |
| --- | --- |
| Multiply by 4 for compressor surge: | _____ W |
| MINIMUM INVERTER SURGE RATING: | _____ W |

## YOUR COMPLETE SYSTEM REQUIREMENTS

| Component | Your Specification |
| --- | --- |
| System Voltage | _____ V |
| Daily Energy Consumption | _____ Wh/day |
| Daily Amp-Hours | _____ Ah/day |
| Battery Bank Capacity | _____ Ah |
| Solar Array Size | _____ W |
| Charge Controller Rating | _____ A |
| Inverter Size (if needed) | _____ W surge |

## Quick Reference Formulas

Watts = Volts × Amps

Watt-hours = Watts × Hours

Amp-hours = Watt-hours ÷ System Volts

Battery Ah = (Daily Ah × Days of Autonomy) ÷ Depth of Discharge

Solar Watts = (Daily Wh ÷ Peak Sun Hours) ÷ System Efficiency

## Notes and Considerations

Use this space to record specific model numbers, vendor information, installation notes, or modifications to your plan:

____________________________________________________________________________________________________________________________________________

____________________________________________________________________________________________________________________________________________

____________________________________________________________________________________________________________________________________________

____________________________________________________________________________________________________________________________________________

____________________________________________________________________________________________________________________________________________

____________________________________________________________________________________________________________________________________________